Customer Product Quality Requirements Form

Part Number: 1525		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

# 1525	BUFFER SOLUTION

pH 10.00 +- 0.01 @ 25 DEG C.

N.I.S.T. TRACEABLE  (BLUE)

NOTE 1: ONCE THE BUFFER HAS BEEN APPROVED FOR PACKAGING, IT MUST BE  PACKAGED  WITHIN 1 DAY AFTER APPROVAL.

NOTE 2: DO NOT ADD COLOR TO THE BUFFER UNTIL THE DAY BEFORE IT WILL BE PACKAGED TO PREVENT STAINING OF THE TANK.

1-LITER FORMULATION

To 1 liter of clear 10 buffer (1526) that has been checked at least two consecutive days and passed NIST ph verification, add 0.0040 grams of Bromothymol blue sodium salt (B7113) and stir until homogenous. The buffer will turn a deep blue color.

REVISION: 2.4	DATE: 03/26/2019   	  INITIAL: STN

EXPIRATION:  2 YEARS

FILTERING INSTRUCTIONS:  THIS BATCH IS USUALLY MADE IN A LARGE TANK AND IS FILTERED

THROUGH 75UM INLINE FILTER.  IF THIS BATCH IS MADE IN A SMALLER TANK USE GLASS WOOL

TO REMOVE ANY STIR BAR DEBRIS.

SPECIAL PACKAGING INSTRUCTIONS: THIS MUST BE PACKAGED 1 DAY AFTER APPROVAL.

PACKAGING:  CLEAR PLASTIC OR CUBES

HMIS CODE:  10OB

PERSONAL PROTECTION:  GLOVES, GLASSES

STANDARDIZATION: USE NIST TRACEABILITY PORTION OF QCSP20B TO STANDARDIZE TO 10.00 +- 0.02 @ 25 DEG C.

APPEARANCE CLEAR BLUE ODORLESS LIQUID

DISPOSAL: GENERAL

REVISION 1.0: INCREASED AMOUNT OF  BROMOTHYMOL BLUE TO 0.004 GPL FROM 0.001 GPL TO TRY TO INCREASE RETENTION OF COLOR.

REVISION 2.0, REVISED TO INCLUDE PART NUMBERS FOR RAW MATERIALS AND TO VERIFY/CHANGE SIZES TO MATCH OPEN SYSTEMS

REVISION: 2.1 10/30/01 REVISED TO PUT THAT THIS SOLUTION MUST STIR FOR 1-2 FULL DAYS AND

PASS TWO CONSECUTIVE TEST BEFORE PACKAGING AND THAT IT MUST BE PACKAGED WITHIN 1 DAY

AFTER  APPROVAL.AND ADD THE COLOR AFTER YOU DILUTE TO 1 LITER AND RIGHT BEFORE PACKAGING..

Revision 2.2, 08/15/03: revised this product to be made from 1526, clear 10 buffer since the clear product is always made first. STN

Revision 2.3 04/27/11: added filtering instructions. RK

Revision 2.4, 03-26-2019: updated description specification to be +- 0.01 from +- 0.02. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /