Customer Product Quality Requirements Form

Part Number: 2169		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

2169        	           COMBINED COLOR REAGENT

APHA FOR CHLORIDE ANALYSIS

RINSE VESSEL WITH DEIONIZED WATER BEFORE USE.

PREPARE THIS SOLUTION VOLUMETRICALLY, USING CLASS A PIPETS.

1  LITER  FORMULATION

FOR EACH LITER OF FINAL SOLUTION,  INTO A 1 LITER VOLUMETRIC FLASK, PIPET 150.0 MLS OF FERRIC NITRATE SOLUTION (BET005S1) AND 150.0 MLS OF MERCURIC THIOCYANATE SOLUTION FOR CHLORIDES (BET005S2). DILUTE TO 1 LITER WITH DEIONIZED WATER (9765) AND STIR UNTIL HOMOGENOUS.

REVISION:  0.2 	DATE: 06-09-2014		INITIAL:  _STN

EXPIRATION: 1 YEAR

SPECIAL PACKAGING INSTRUCTI0NS:  THIS PRODUCT MUST BE FILTERED THROUGH FINE  0.22 OR 0.45 UM FILTER PAPER. WEAR GLOVES, THIS SOLUTION IS TOXIC!

PACKAGING: 1 liter amber plastic, 4 liter amber plastic, 500 ml amber plastic, 10liter cube

Retain: Amber plastic

HMIS CODE:  310C, TOXIC, FLAMMABLE LIQUID

PERSONAL PROTECTION: GLOVES, GOGGLES, LABCOAT

STANDARDIZATION: THIS PRODUCT MUST BE FILTERED THROUGH FINE 0.22 OR 0.45 UM FILTER PAPER DURING PACKAGING

APPEARANCE OF FINAL SOLN/ ODOR: CLEAR BROWN LIQUID WITH MILD ALCOHOL ODOR.

DISPOSAL:   METALS WASTE ONLY, CONTAINS MERCURY!

REVISION 0.0 05/03/01: CREATION DATE FOR BATCH CARD. MG

REVISION 0.1: 01/14/13, CHANGED PACKAGING FROM CLEAR PLASTIC TO AMBER PLASTIC STN

REVISION 0.2, 06-09-2014: ADDED FILTERING INSTRUCTION PER STANDARD M ETHODS.

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /