Customer Product Quality Requirements Form

Part Number: VUL249C		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

### VUL249	ETHYLENEDIAMINE DIHYROCHLORIDE

1.0 MOLAR SOLUTION

Rinse vessel with deionized water before use.

Inspect stir bar and remove any traces of metal before use

Wear gloves and goggles when making this batch.

1 Liter formulation

For each liter of final solution, adjust based on assay and dissolve 133.03 grams of Ethylenediamine Dihydrochloride (E6895)  into about 800 mls of deionized water and stir until completely dissolved. Dilute to 1 liter with deionized water (9765) and stir until homogenous.

Use the following equation to verify molarity of 1.000 +- 0.005 Molar:

Molarity = (Act gm E6895 x assay) / 133.03 / final volume in liters /100

Revision:  0.4          Date:  06/20/2019          Initials:  STN

Expiration: 1 Year

Special posting instructions: put calculated molarity to 3 decimal places on COA

Special labeling instructions: Do not put calculated molarity on label.

Special filtering instructions: can be glass wool, y-strainer or paper filtered.

Special Packaging: None

Packaging: VUL249 = 4 Liter Cube;

VUL249B = 4 Liter Clear Plastic; VUL249C= 10 Liter Cube

HMIS Code: 200C

Personal Protection: Gloves and Goggles

Standardization: Density to 1.04497 ± 0.00103@ 20°C (1.04394- 1.04600). Use above equation to verify molarity of 1.000 +- 0.005 Molar

Appearance/Odor: Clear Water White Liquid / None

Disposal: General Waste

Revision: 0.0, 09/14/06, Creation date for batch card. STN

Revision: 0.1, 10/20/15, Added specifications for density. DEJ

Revision 0.2, 10/11/17: Updated density specs. DL

Revision 0.3, 12-05-2017: added verification calculation. STN

Revision 0.4,06/20/19: updated density information based on at least 8 data points. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /