Customer Product Quality Requirements Form

Part Number: ODP102		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

ODP102	SODIUM ACETATE

REAGENT #5

RINSE VESSEL WITH DEIONIZED WATER BEFORE USE.

WEAR GLOVES AND GOGGLES WHILE MAKING THIS BATCH.

1 LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION, DISSOLVE  550.0 GRAMS OF SODIUM ACETATE TRIHYDRATE (S2412), OR 331.5 GRAMS OF REAGENT GRADE SODIUM ACETATE ANHYDROUS  (S2420) INTO ABOUT 400 MLS OF DEIONIZED WATER (9765). DILUTE TO ABOUT 900 MLS WITH DEIONIZED WATER AND STIR.  NEXT, ADD 50 MLS OF REAGENT ACS GRADE ACETIC ACID (A1100) AND STIR UNTIL ALL OF THE SOLIDS DISSOLVE.  ONCE COOLED TO ROOM TEMPERATURE,  DILUTE TO 1 LITER WITH DEIONIZED WATER AND STIR UNTIL HOMOGENOUS.

REVISION:  0.2	    DATE: 07/08/2019	                     INITIAL:STN

EXPIRATION:  1 YEAR

SPECIAL PACKAGING: NONE

PACKAGING:   ODP102= 4 LITER CUBE, ODP102A=4 LITER JUG, ODP102B= 10 Liter cube

HMIS CODE:   210C

PERSONAL PROTECTION: GLOVES, SPLASH GOGGLES, LABCOAT.

STANDARDIZATION:  USE DENSITOMETER TO VERIFY DENSITY OF 1.16025±0.00692@20°C (1.15334-1.16717)

APPEARANCE / ODOR:  CLEAR  WATER WHITE / ACETIC ACID SMELL

DISPOSAL:  GENERAL

REVISION 0.0, 08-06-98: CREATION DATE FOR BATCH CARD. STN

REVISION 0.1, 10-16-2015: ADDED DENSITY CHECK INFORMATION. STN

Revision 0.2,07/08/19: Updated density information using at least 8 data points. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /