Customer Product Quality Requirements Form

Part Number: OXY1479		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

OXY1479           BARIUM DIPHENYLAMINESULFONATE INDICATOR

(BDAS INDICATOR) FOR THE TITRATION ANALYSIS OF SODIUM CHLORATE

RINSE VESSEL WITH DEIONIZED WATER BEFORE USE.  INSPECT STIR BAR FOR METAL AND REMOVE IF NECESSARY!!! NOTE THIS IS A CUSTOMER SUPPLIED METHOD AND CANNOT BE ALTERED WITHOUT THE PERMISSION OF OCCIDENTAL!THIS SOLUTION MUST BE MADE VOLUMETRICALLY.

NOTE: DO NOT MAKE MORE THAN 6 LITERS AT A TIME.

1 LITER FORMULATION

FOR EACH LITER OF FINAL SOLUTION, DISSOLVE 1.500 GRAMS OF BARIUM DIPHENYLAMINESULFONATE, INDICATOR (B0501) GRADE INTO 1,000 MLS OF REAGENT ACS GRADE PHOSPHORIC ACID. (P2380) STIR UNTIL DISSOLVED AND HOMOGENOUS.

USE THE FOLLOWING EQUATION TO VERIFY 1.5000 +- 0.0005 GPL B0501:

GPL = (ACT GM B0501 / FINAL VOLUME IN LITERS)

REVISION:  0.2	    DATE:  10/13/2017	                     INITIAL:_STN

EXPIRATION: 1 YEAR

SPECIAL POSTING INSTRUCTIONS: PUT CALCULATED GPL B0501 TO 4 DECIMAL PLACES ON

LABEL

SPECIAL LABELING INSTRUCTIONS: DO NOT POUT CALCULATED VALUE ON LABEL.

FILTERING INSTRUCTIONS: NONE

SPECIAL PAGAGING INSTRUCTIONS: NONE

PACKAGING:  OXY1479 = 1 LITER AMBER GLASS, OXY1479A = 250 L AMBER PLASTIC WITH FLIP

TOP CAP

HMIS CODE:  300C, CORROSIVE

PERSONAL PROTECTION: GLOVES, SPLASH GOGGLES, LAB JACKET.

STANDARDIZATION:  USE THE ABOVE  EQUATION TO VERIFY 1.5000 +- 0.0005 GPL B0501

APPEARANCE OF FINAL SOLN/ ODOR: PALE BLUE LIQUID/ NONE  ________________INIT:

DISPOSAL: ACID WASTE FOR MORE THAN 1 LITER.

REVISION 0.0: 12/26.97,  CREATION DATE FOR BATCH CARD

REVISION 0.1: 09/05/14, ADDED THE INSTRUCTIONS TO MAKE THE SOLUTION VOLUMETRICALLY  AND TO MAKE NO MORE THAN 6 LITERS AT A TIME. UPDATED TO NOTE9 STN

REVISION 0.2, 10/13/2017: ADDED VERIFICATION CALCULATION. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /