Customer Product Quality Requirements Form

Part Number: 6457		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

6457                                  Nitric Acid

25% v/v Solution

Rinse vessel with deionized water (9765) before use.

Wear gloves (At least one of the chemicals in this product will permeate nitrile gloves. You must wear a barrier glove under a larger nitrile glove. Remember, the barrier glove is not a disposable glove.) and goggles while making this batch.

1-liter formulation

For each liter of final solution, add 250 mls of Reagent ACS grade Nitric acid (N3805) into about 600 mls of deionized water (9765).  Dilute to 1 liter with deionized water (9765) and stir until homogenous.

Use the following equation to verify 25.0 +- 0.1% v/v nitric acid:

v/v% = (act mls nitric acid / final volume in liters / 10)

Revision:  0.1	                             Date: 07-19-2017                   Initial: STN

Expiration:   1 year

Special posting instructions: put calculated v/v% nitric acid to 1 decimal place on COA

Special labeling instructions: do not put calculated value on label.

Special filtering instructions: can be glass wool, paper or y-strainer filtered

Special Packaging Instructions:  Wear gloves and goggles

Packaging:  500 ACID BOTTLE

HMIS Code:   300C CORROSIVE

Personal Protection:  Gloves, goggles

Standardization: Use above equation to verify 25.0 +- 0.1% v/v nitric acid

Appearance Of Final Soln/ Odor:  Clear water white / Odorless

Disposal:   General

Revision 0.0 03/23/04:  Creation date for batch card.  STN

Revision 0.1, 07-19-2017: added verification calculation. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /