Customer Product Quality Requirements Form

Part Number: OXW001		    X      New Product       	               Revised Product

Company: OXY Whichita		Contact:  Dale Jones

Address:

Phone:					email:  Dale_Jones@oxy.com

*If not already indicated, please specify if this product needs to be prepared or verified to an

industry standard procedure as ASTM, EPA, APHA or other method:

Note: in the event that the above method becomes revised, it will be the customer’s responsibility to provide updated information / revision requests to Aqua Solutions.

OXW001	Ammonium Acetate 560 GPL (±1GPL)

Refer to each raw material SDS before handling the material for the first time.

Wear gloves (Nitrile), and goggles during production.

Rinse vessel with deionized water (9765)  before use.

Inspect stir bar and remove any traces of metal before use.

1 Liter formulation approximate

For each liter of final solution, dissolve 560.0 grams of reagent ACS grade Ammonium Acetate (A4305) into about 600 mls of deionized water (9765). Stir until all of the ammonium acetate dissolves. Dilute to 1 liter with deionized water and stir until homogenous.

Use the following equation to verify GPL 560±1:

GPL= Act gms A4305/ Final Volume in liters

Revision:  0.0                           Date: 11/14/2019	Initial: STN

Expiration: 18 months

Special posting instructions:  None

Special Labeling Instructions: None

Special Packaging:   Wear gloves (Nitrile) and goggles

Special Filtering Instructions: None.

Packaging:  10 Liter Cube .

Retain: clear plastic

HMIS CODE: 200C

Packager Personal Protection: Gloves, goggles

Technician Personal Protection:  Gloves, goggles

Standardization: Use the above equation to verify GPL 560±1

Appearance / Odor: Clear liquid   / ammonia or vinegar odor          Initial :_________

Disposal:  Organic waste

Revision 0.0 11/14/19: Creation date for batch card. STN

***Please make any revisions, sign and email or fax back to Aqua Solutions @ 281-479-1833

Customer Authorization Signature:  ________________________________________  Date:         /           /

Laboratory Approval: __________________________________________________  Date:       /         /